ESTEC Contract No 4000103890/11/NL/CB

“ASN.1 Space Certifiable Compiler Extensions”

«ACN User Manual»

issued

2016-02-14

Document Ref:		ACN-UM

Global Id:		NRP-SPACE-ESA-4000103890-ACN-UM				Version:		3.2

Editing tool:		Microsoft Word 2007

Authors:		George Mamais (g_mamais@neuropublic.gr)

Maxime Perrotin (maxime.perrotin@esa.int)

Reviewers:		Thanassis Tsiodras (ttsiodras@gmail.com)

Neuropublic SA

Aitolikou & 11 Sfaktirias Str, 185 45 Piraeus, GREECE Phone: +30 210 4101010 Fax: +30 210 4101013

Table of Contents

Acronyms and Abbreviations	3

1.What is ACN?	4

2.Short introduction to ACN	5

3.ACN Encoding Properties	7

3.1.size property	7

3.1.1.Fixed form	7

3.1.2.Variable size with length specified in external field	7

3.1.3.Null terminated	8

3.2.encoding property	8

3.3.endianness property	9

3.4.align-to-next property	9

3.5.encode-values property	10

3.6.true-value and false-value properties	10

3.7.present-when property	10

3.8.determinant property	12

3.9.mapping-function property	12

4.Enhanced Options	14

4.1.Fields introduced in the ACN grammar	14

4.2.Parameterized encodings and deep field access	14

4.2.1.Length determinant is below current node	15

4.2.2.Length determinant is above current node	16

4.2.3.Length determinant is in completely different subtree	17

# Acronyms and Abbreviations

| ANSI | American National Standards Institute |
| --- | --- |
| API | Application Programming Interface |
| ASN.1 | Abstract Syntax Notation 1 |
| ASN1scc | ASN.1 space certifiable compiler |
| AST | Abstract Syntax Tree |
| BER | Basic Encoding Rules |
| EAST | Enhanced Ada SubseT |
| ECN | Encoding Control Notation |
| ESA | European Space Agency |
| ESTEC | European Space Research and Technology Centre |
| PER | Packed Encoding Rules |
| XML | eXtensible Markup Language |
| XSD | XML Schema Documentation |

# What is ACN?

ASN.1 is a language for defining data structures (i.e. messages) in an abstract manner. An ASN.1 specification is independent of the programming language (C/C++, Ada etc), the hardware platform or even the encoding method used to serialize the defined messages. The encoding mechanism, i.e. how bits and bytes are written over the wire, is determined by the ASN.1 encoding. Although the standardized ASN.1 encodings may offer some important benefits such as speed and compactness for PER or decoding robustness for BER, there is no way for the designer to control the final encoding (i.e the format at the bit level). This is a problem for situations where there is legacy binary protocol and we must replace one of the communicating parties using ASN.1 encoders/decoders (i.e. when the other, legacy system, must remain unchanged).

ACN is a proprietary ASN.1 encoding which addresses the above need: it allows protocol designers to control the format of the encoded messages at the bit level.

The main features of ACN are:

- Easy to learn, with simple and clear syntax but also with enough power to cover complex cases

- Encoding instructions are written in a separate file, so that the original ASN.1 grammar remains unpolluted

- Fields which do not carry semantic information but are used only during the decoding process (e.g. length fields, choice determinants etc) may either appear in the ASN.1 grammar or introduced only in the ACN specification.

The sections that follow showcase ACN through easy to follow code examples.

# Short introduction to ACN

Every ASN.1 type has a set of encoding properties that can be set in order to achieve the desired binary encoding. These properties control certain aspects of the encoding process such as: the size of type being encoded, how values are encoded (twos-complement vs positive integer encoding, etc), the presence/absence of a certain field etc.

These properties are assigned to ASN.1 types using a pair of square brackets (“[“ and “]”) as seen in Listing 2. The encoding properties assignment is carried out in a separate file – the ACN file, so that the original ASN.1 grammar remains “clean” from encoding specifications.

Here is a simple ASN.1 grammar:

MYMOD DEFINITIONS AUTOMATIC TAGS::= BEGIN

MyInt ::= INTEGER (-100 .. 100)

MyInt2 ::= INTEGER (0 .. 1000)

MySeq ::= SEQUENCE {

a1 INTEGER (1..20),

a2 INTEGER (-10 .. 20),

a3 MyInt,

a4 MyInt2

}

END

Listing 1: Sample ASN.1 grammar

and here is an example ACN encoding for this grammar:

MYMOD DEFINITIONS ::= BEGIN

--ACN allows constant definitions

CONSTANT WORDSIZE ::= 32

--We can make basic math with ACN constants

CONSTANT LARGEST-INT ::= 2^^(WORDSIZE - 1)-1

--MyInt will be encoded as twos complement integer.

--Size will be 1 byte

MyInt[size 8, encoding twos-complement]

-- If no encoding properties are present, then

-- encoding properties will be automatically populated

-- so that the behavior matches the one of uPER i.e.

-- size 10, encoding pos-int

MyInt2 []

-- encoding properties for types defined

-- within constructed types (i.e. fields)

MySeq [] {

a1 [],

a2 [size 32, encoding twos-complement, endianness little],

a3 [],

a4 []

}

END

Listing 2: Sample ACN grammar for the ACN grammar of Listing 1

By looking at the above code example, we see the following:

- For each ASN.1 module there is one ACN module with the same name.

- We can optionally define some integer constant values (WORDSIZE, LARGEST-INT etc) which can be referenced by the rest of the ACN specification.

- The ACN module contains the types (i.e. the type references) declared in the ASN.1 module followed by the encoding properties.

- The encoding properties may be absent. (The pair of open close brackets [ ] must be present though). In this case, the encoding properties have values which are calculated as follows:

  - Referenced types inherit the properties of their base types

  - For non referenced types (or referenced types whose base types have no encoding properties), the encoding properties are automatically populated with such values as to mimic the behavior of uPER.

- For types declared within constructed types such as SEQUENCE / CHOICE / SEQUENCE OF, the encoding properties are declared after the component names

- The encoding properties are declared at type reference level. If a new type is declared in the ASN.1 grammar based on an existing type reference, then the new type inherits from the base type its encoding properties.

# ACN Encoding Properties

## size property

The size encoding property controls the size of the encoding type. It comes in three forms:

### Fixed form

This form is used when the size of the encoded type is fixed and known at compile time

Syntax

size intExpr – the units are provided in the table below

Example

size 10

size WORDSIZE/2	-- WORDSIZE is an ACN constant defined before

The following table lists the ASN.1 types where the fixed form can be applied as well as the corresponding count unit.

| Asn1 Type | Count unit of intExpr |
| --- | --- |
| Integer | Bits |
| Enumerated | Bits |
| Bit String | Bits |
| Octet String | Octets |
| IA5String | Characters |
| Numeric String | Characters |
| Sequence/set Of | Elements of sequence/set of |

Table 1: ASN.1 types where the size property can be applied

### Variable size with length specified in external field

This form of size property is functionally equivalent with the previous one. The main difference is that the length field is an external field provided in the ACN grammar

Syntax

size field

Example

| size length | length is an integer field defined in the same scope with the encoded type |
| --- | --- |
| size header.length | header is a sequence type defined in the same scope with the encoded type and which contains an integer type component named length |

This form of size property can be applied to bit string, octet string, character strings and sequence/set of types.

### Null terminated

This form is applicable only for IA5String and Numeric string types. In this case, the end of the string is determined by the presence of a null character (0). User may define the null terminated character with termination-pattern encoding property. Here are some examples:

MyPDU ::= IA5String(SIZE(20))(FROM("A".."Z"|"a".."z"|" "))

Listing 3: Sample ASN1 grammar to demostrate size null-terminared property

MyPDU[encoding ASCII, size null-terminated]

-- ASCII encoding, null terminated

MyPDU[encoding ASCII, size null-terminated, termination-pattern '01'H]

-- ASCII encoding, null terminated, the termination pattern is the character 0x01

Listing 3: Sample ACN grammar to demostrate size null-terminared property

## encoding property

The encoding property can be applied only to integer, enumerated, real, IA5String and Numeric string types.

Syntax

encoding encvalue

where encvalue is one of pos-int, twos-complement, BCD, ASCII, IEEE754-1985-32 and IEEE754-1985-64

Example

encoding pos-int

encoding BCD

| Encoding value | Applicable ASN.1 types | Remarks |
| --- | --- | --- |
| pos-int | Integer, enumerated | The ASN.1 integer must have constraints so that only positive values are allowed. Otherwise the compiler will report an error. |
| twos-complement | Integer, enumerated |  |
| ASCII | Integer, enumerated, IA5String, NumericString | The ASCII code of the sign symbol (‘+’ or ‘-‘) is encoded first (mandatory) followed by the ASCII codes of the decimal digits of the encoded value. For example, the value 456 will be encoded in the four ASCII codes: 42 (i.e. ‘+’), 52, 53, 54. |
| BCD | Integer, enumerated | The ASN.1 integer must have constraints so that only positive values are allowed. Otherwise the compiler will report an error. |
| IEEE754-1985-32 | Real | http://en.wikipedia.org/wiki/IEEE_754-1985 |
| IEEE754-1985-64 | Real | (same link as above) |

Table 2: ASN.1 properties where the encoding property can be applied

## endianness property

The endianness property can be applied only to fix size integers (and in particular when the size is 16, 32 or 64 bits), enumerated and real types and determines the order of the encoded bytes. For more information please refer to http://en.wikipedia.org/wiki/Endianness

Syntax

endianness endianness-value

where endianness-value is big or little

Example

endianness little

endianness big (Default)

| Encoding value | Applicable ASN.1 types | Remarks |
| --- | --- | --- |
| Big | Integer, enumerated, Real | The 32 bit integer value 0xAABBCCDD will be transmitted as follows: 0xAA, 0xBB, 0xCC, 0xDD |
| Little | Integer, enumerated, Real | The 32 bit integer value 0xAABBCCDD will be transmitted as follows: 0xDD , 0xCC , 0xBB, 0xAA |

Table 3: endianness property description

## align-to-next property

This property can be applied to any ASN.1 type, and allows the type to be encoded at the beginning of the next byte or word or double word of the encoded bit stream.

Syntax

align-to-next alignValue

Example

align-to-next byte  -- 8 bits

align-to-next word – 16 bits

align-to-next dword – 32 bits

## encode-values property

This property can be applied only to enumerated types and controls whether the enumerant values will be encoded or their indexes. When present, the values (not indexes) of enumerants will be encoded.

Example

[encode-values]

## true-value and false-value properties

These two mutually exclusive properties can be applied only to Boolean types and determine what value will be used to encode TRUE or FALSE values.

Syntax

true-value bitStringValue

false-value bitStringValue

Example

true-value ‘111’B

false-value ‘0’B

## present-when property

The present-when property is used in optional SEQUENCE components and in CHOICE alternatives

In the case of OPTIONAL components the syntax is as follows:

Syntax

Present-when booleanFld

where booleanFld is a reference to a boolean field

Example

MySeq ::= SEQUENCE {

alpha 	INTEGER,

gamma		REAL      OPTIONAL

}

Listing 3: Sample ASN.1 grammar

MySeq[] {

alpha [],

beta  BOOLEAN  [],

gamma [present-when beta, encoding IEEE754-1985-64]

}

Listing 4: ACN grammar for ASN.1 grammar of Listing 3

In the above example, gamma field is present only when beta is TRUE.

In the case of CHOICE alternatives the syntax is as follows

Syntax

Present-when fld1==val1 fld2==val2 ... fldn==valn

where fldi is a reference to a an integer or string field and vali is constant integer or string value.

Example

MYMOD DEFINITIONS AUTOMATIC TAGS::= BEGIN

COLOR-TYPE	::= INTEGER (0..255)

COLOR-DATA ::=	CHOICE {

green INTEGER (1..10),

red 	INTEGER (1..1000),

blue 	IA5String (SIZE(1..20))

}

MySeq ::= SEQUENCE {

colorData COLOR-DATA

}

END

Listing 5: Sample ASN.1 grammar

MYMOD DEFINITIONS ::= BEGIN

COLOR-TYPE [encoding pos-int, size 8]

MySeq [] {

activeColor1 COLOR-TYPE [],

activeColor2 COLOR-TYPE [],

colorData 	<activeColor1, activeColor2> []

}

COLOR-DATA<COLOR-TYPE:type1, COLOR-TYPE:type2> [] {

green [present-when type1==1  type2==10],

red   [present-when type1==20 type2==20],

blue  [present-when type1==50 type2==20]

}

END

Listing 6: ACN grammar for ASN.1 grammar of Listing 3

## determinant property

The determinant property is an alternative (simpler) way to determine which choice alternative is encoded. The encoded choice alternative is determined by an external enumerated field which must have the same names in its enumerants as the names of the choice alternatives.

Syntax

determinant enumFld

Example

MYMOD DEFINITIONS AUTOMATIC TAGS::= BEGIN

RGB ::= ENUMERATED {green, red, blue}

MySeq ::= SEQUENCE {

beta 		BOOLEAN,

colorData	CHOICE {

green REAL,

red 	INTEGER,

blue 	IA5String (SIZE(1..20))

}

}

END

Listing 7: Sample ASN.1 grammar

MYMOD DEFINITIONS ::= BEGIN

MySeq [] {

activeColor RGB [],

beta 		[],

colorData 	[determinant activeColor]

}

END

Listing 8: ACN grammar for ASN.1 grammar of Listing 7

In the example above, the active alternative in colorData choice is determined by the enumerated field activeColor.

## mapping-function property

The mapping-function property allows to apply a filter to integer values, with the possibility to provide the filter function in the target language (C or Ada). For example in the MIL-1553 encodings, the length determinants of size (1..32) arrays are encoded using 5 bits and the size 32 is encoded with value 0 (i.e. 0 means 32 elements):

MYMOD DEFINITIONS AUTOMATIC TAGS::= BEGIN

LENG-DET ::= INTEGER (1..32)

WORD ::= INTEGER (0..65535)

Milbus ::= SEQUENCE {

words32 SEQUENCE (SIZE(1..32)) OF WORD

}

END

Listing 9: Sample ASN.1 grammar

MYMOD DEFINITIONS ::= BEGIN

LENG-DET [encoding pos-int, size 5, mapping-function milbus2]

WORD[]

Milbus[] {

length LENG-DET [],

words32 [size length]

}

END

Listing 10: ACN grammar

The user in that case has to provide the code for the filter function milbus2 (either in C or in Ada):

asn1SccSint milbus2_encode(asn1SccSint val) {

return val == 32 ? 0 : val;

}

asn1SccSint milbus2_decode(asn1SccSint val) {

return val == 0 ? 32 : val;

}

Listing 11: C filter function for milbus2 encoding

When calling the ASN.1 compiler, use the -mfm flag. Assuming the functions are defined in mapFunctions.c:

$ asn1.exe -c -ACN -o c_out/ -atc -mfm mapFunctions a.asn1 a.acn

Listing 12: Invocation of ASN1SCC with custom filter functions

For convenience, the ASN1SCC runtime provides pre-defined filter functions. In particular for the above example the mapping-function milbus is available off-the-shelf. Others could be added on request if frequently used.

# Enhanced Options

## Fields introduced in the ACN grammar

In some cases, the value for the encoding properties “size”, “present-when” and “determinant-tag” may be another field. These fields do not carry semantic (i.e. application specific) information but are used only in the decoding and encoding processes. Therefore these fields may not exist in the ASN.1 grammar but introduced only in the ACN one. For example, Listing 3 can be modified as follows:

MySeq ::= SEQUENCE {

alpha 	INTEGER,

gamma		REAL       OPTIONAL

}

Listing 13: The revised ASN.1 grammar of Listing 3. Field ‘beta’ is missing.

Seq[] {

alpha [],

beta BOOLEAN [],  -- exists only in the ACN file, not the ASN.1 one

gamma [present-when beta, encoding IEEE754-1985-64]

}

Listing 14: Revised ACN grammar for the ASN.1 grammar of Listing 14. Field ‘beta’ along with the type (BOOLEAN) is introduced.

Please notice that field ‘beta’ does not exist in the ASN.1 grammar but it was introduced only in the ACN grammar.

Another way to introduce fields (e.g. for alignment, or to add a standard-imposed data pattern in the encoding) is to use the NULL construct with the “pattern” encoding:

T-tc-packetID []

{

ccsds-version-number     NULL  [pattern '000'B],

packet-type              NULL  [pattern '1'B] ,

has-data-fieldhdr        NULL  [pattern '1'B],

apid []

}

## Parameterized encodings and deep field access

There are cases where the length field of a sequence of (or choice determinant, or optionality determinant etc) is not at the same level (i.e. components of a common parent) as the sequence of itself. Actually there are three distinct cases:

- The length determinant is one or more levels more deeply than the SEQUENCE OF

- The SEQUENCE OF is one or more levels more deeply than the length determinant

- The length determinant and the SEQUENCE OF are located in completely different nodes which just have a common ancestor.

These three cases are explained in more detail in the following sub-paragraphs

### Length determinant is below current node

This case is illustrated in Figure 1. Field secondaryHeader, which is optional, is present when the secHeaderFlag in the primaryHeader is true.

- Deep field access – case a.

The corresponding ASN.1 / ACN grammar is:

--ASN.1 DEFINITION

Packet ::= SEQUENCE {

primaryHeader SEQUENCE {

version INTEGER,

seqNr	 INTEGER,

secHeaderFlag	 BOOLEAN

},

secondaryHeader SEQUENCE {...} OPTIONAL

}

-- Encodings definition

Packet  {

primaryHeader[] {

version [],

seqNr [],

secHeaderFlag []

}

secondaryHeader [present-when primaryHeader.secHeaderFlag]

}

Listing 15: ACN grammar demonstrating access to fields at different levels

As shown in the example above, to access a “deep field” located in a child structure we follow the C language notation i.e. fieldname.fieldname.fieldname etc. until we reach the field we want.

### Length determinant is above current node

This is the case where the array (sequence of_ is one or more levels more deeply than the length determinant. For example, field “nrCalls”, which is a top level field, contains the number of calls in the array “calls” located under “SourceData”. Obviously, the “nrCalls” field is not accessible from the “calls” field. To overcome this issue, we must make the SourceData structure parameterized. This case is shown in Figure 2.

- Deep field access – case b.

The corresponding ASN.1 / ACN grammar is:

--ASN.1 DEFINITION

TAP2File ::= SEQUENCE {

nrCalls INTEGER,

data SourceData

}

SourceData ::= SEQUENCE {

operatorID IA5String,

calls SEQUENCE (SIZE(1..100))OF Call

}

--ACN DEFINITION

TAP2File {

nrCalls [],

data <nrCalls> [ ] -- nrCalls is passed as a parameter in SourceData

}

SourceData<INTEGER:nElements>

-- nElements is a parameter used in encoding/decoding

-- passed in from the levels above (in this case, TAP2File level)

{

operatorID [],

calls[size nElements] -- points to a parameter not a field

}

Listing 16: ACN grammar demonstrating parameterized encodings

Please note the “<>” in the encoding definition of the SourceData which contains the list with the encoding parameters (in this example, just one).

### Length determinant is in completely different subtree

This case is the combination of the two previous cases. A typical case is depicted in Figure 3. In this example, field “nCalls”, which is located under “header” record, contains the number of calls in the “calls” array under “SourceData”.

Field nCalls (length determinant) and field calls (the SEQUENCE OF) are components of two sibling structures (Header, SourceData) and have no access to each other.

- Deep field access – case c.

To handle this case, we must apply the techniques of both previous cases. The corresponding ASN.1 / ACN grammar would be:

--ASN.1 DEFINITION

TAP3File ::= SEQUENCE {

header Header,

data SourceData

}

Header ::= SEQUENCE {

operatorID IA5String,

nCalls INTEGER

}

SourceData ::= SEQUENCE {

calls SEQUENCE (SIZE(1..100)) OF Call	-- length field is contained in the header.nCalls

}

--ACN DEFINITION

TAP3File {

header [] {},

data <header.nCalls> [ ] -– header.nCalls is passed as a parameter

-- in SourceData

}

Header[]{

operatorID[],

nCalls[]

}

SourceData<INTEGER:nElements> 	-- parameters

{

calls[size nElements] – “size” points to a parameter, not a field

}

Listing 17: ACN grammar demonstrating parameterized encodings and deep field access